This is my attempt to produce an up  to date F4U1 Birdcage in 1/72nd scale. Hasegawa produces a nice little kit of the Birdcage Corsair but the kit suffers from old time engraved panel lines. I decided to use similarity of the Academy 1/72 Corsair (an recessed panel line copy of the Hasegawa rendition) to build my Birdcage.

The Academy Corsair will provide 95% of plastic parts. As the Academy shares the same faulty cockpit as the Hasegawa model, I replaced with a True Detail resin set. It correctly depicts an early floorless Corsair cockpit. The bottom of the resin parts is removed as a see through. Early Corsairs had a window under fuselage as a common measure in the US NAVY of the time (F2A Buffaloes and F4F Wildcats had it also) to improve visibility during carrier recovery. Main colors are interior green, with black instrument board and throttle cadrant. Some white and red details are spread in cockpit on various knobs (be light on colors, you try not to build a christmas tree here).

Fuselage is cut behind cockpit. This part is replaced by the corresponding part cut from the Hasegawa Birdcage. Part is cemented into place, joint is filled and filed, then panel lines are rescribed for the part to blend in the kit general aspect.

A hole is cut in the bottom of fuselage for window. Window is cut from either an old transparent part, or vacuum formed using rodhoid or even taken from a Tamiya F4U1D if you don't plan to use the part (that is actualy what I did).

Engine is detailed painting cylinders aluminium with a wash of diluted Tamiya smoke for an oily and shadowed aspect. Crankcase is painted medium grey. Stretched sprue is used to depict wiring.

Construction is straightforward.

* cooling vents are thinned from the inside of engine cowling

* tail gear is shortened by a good 1/8'' giving a finished aircraft a pronounced nose up stance

* boarding step (slot) in inner right wing flap is filled

* position, identification, landing and formation lights are built according to documentation to depict standard lighting of the version.

* exhausts are replaced with hollow stretched sprue.

* wiring is added to landing gears to ressemble brake lines.

* Hi Tech resin wheels are used.

I painted kit with Humbrol enamel parts : A thin layer of chrome silver was airbrushed on fuselage. I then painted bottom surfaces gull grey, upper surfaces were a standard blue grey of the time. Iused several hues of the same color to show a panel weathering. Black wash was used to enhance control surfaces seperation, a medium brown/grey was used for panel lines. I then gloss varnished, decaled then matt varnished. I then used tip of a sharp blade to scratch some paint to depict paint chipping (surfaces leading edge, cockpit sides, etc…). Decals were taken from a Superscale sheet.