"1/48 Revell Monogram F/A-18D"

by "Jason Cheah"
--------------------

Steve Bamford will insert full sized version of photo 01.jpg here

Tentera Udara DiRaja Malaysia (Royal Malaysian Air Force) received 8 night attack capable F/A-18D Hornets during the course of 1997. The aircraft are tasked with maritime strike and interdiction, and serve with No.18 Sqn. at Butterworth Air Base (an ex-RAAF Mirage III & P-3 Orion base).

This is my model of the TUDM's F/A-18D, built using a 1/48 Revell Monogram F/A-18C which was kit-bashed into a 'D' using mostly parts from the Italeri F/A-18 and a pair of Black Box NACES seats.

Work started off with sawing of part of the R-M kit's spine, and fairing in part of the Italeri kit's twin seat canopy frame/spine assembly with Miliput and Tamiya putty. The front cockpit tub is from the kit, while the rear cockpit tub was also taken from the Italeri kit. Oh yeah, the wheels and a couple of small parts came from Hasegawa.

The rear instrument panel of the TUDM F/A-18D has the same arrangement as that of the USMC Night Attack F/A-18D, and has the corresponding 'stepped' intrument panel coaming. I scratchbuilt the stepped coaming from scrap parts and a lot of Miliput. The 'Night Attack' configuration instrument panel was made from a spare Black Box resin instrument panel chopped up, and its various MFD's, and panels repositioned on a new scratchbuilt panel. The cockpits were sprayed with Gunze Mr.Color No.31 Dark Gray.

| Click on images below to see larger images |  |  |
| --- | --- | --- |
| Steve Bamford will insert a click-able smaller version of photo 03.jpg here. Clicking on that photo will bring up the full sized version of photo 03.jpg | Steve Bamford will insert a click-able smaller version of photo 04.jpg here. Clicking on that photo will bring up the full sized version of photo 04.jpg |  |

I should note that the Italari Hornet has its canopy frames and associated transparent part molded separately, and it took a bit of work to put it together using super glue, applied bit-by-bit along the joint.The resulting joint was sanded lightly when dry. Needless to say, the transparent part needed to be masked beforehand.

| Click on images below to see larger images |  |
| --- | --- |
| Steve Bamford will insert a click-able smaller version of photo 05.jpg here. Clicking on that photo will bring up the full sized version of photo 05.jpg | Steve Bamford will insert a click-able smaller version of photo 06.jpg here. Clicking on that photo will bring up the full sized version of photo 06.jpg |

I decided to drop the trailing edge flaps just to challenge myself (something I regret in retrospect) and proceeded to saw off both. The trailing edge flap slats (for lack of a better word, i.e. the piece covering the gap between the wing and the airfoil shaped part of the flap) was made by carefully sawing the parts off the removed flap and sanding it to thin it down. The slats for the outboard wings were cut from thin plastic card. The resulting 'cut-out' was filled and the airfoil shape of the flaps restored by Miliput blended and sanded to shape. The hinge mechanism were also carefully cut and angled to match.

Steve Bamford will insert full sized version of photo 02.jpg here

Construction of the kit from then on pretty much follows the instructions, but I decided to leave off the horizontal tailplanes until after painting. The slats and the flaps were glued on at an appropriate 'parked aircraft' angle shortly before general painting of the airframe started.

Other features unique to the TUDM Hornet as well as other Late lot F/A-18C/D aircraft were added as below:

1) Twin Counter Measures Dispenser fairings under the intake trunks were scratchbuilt using Miliput and plastic card

2) Raked back UHF/TACAN antenna on spine and under forward fuselage were carved out of plastic card to replace the original

ones.

3) ALQ-165 antenna was removed from front RH nose door, and a replacement made of Miliput was glued to correct location (for a F/A-18D)

4) GPS dome antenna on the spine was added from Miliput sanded to a hemispherical shape.

5) Two ALQ-165 related antennas on the spine were also sculpted out of Miliput

6) Leading edge of centreline pylon was angled slightly with Miliput sanded to shape

| Click on images below to see larger images |  |
| --- | --- |
| Steve Bamford will insert a click-able smaller version of photo 11.jpg here. Clicking on that photo will bring up the full sized version of photo 11.jpg |  |

Once all these details were completed, it was time to spray the aircraft in its unique and characteristic overall FS36118 Gunship Grey paint scheme. Gunze Mr.Color No.305 was used. Following the base colour were the landing gear wells in flat white, rear fuselage exhaust area in silver and the tailpipes in a silver & black paint mixture.

Since Gunze Mr.Color No.305 dries with a Semi-gloss finish, and I had to rush the model done in time for this write up, I decided not to spray a gloss coat.

I used the new F-4DableModels McAir TUDM F/A-18D Hornet decal sheet for this model. Incidentally, I'm also the guy behind this decal sheet which will be featured and reviewed separately.

Unfortunately, there were some areas where silvering appeared, probably due to air bubbles and me rushing to finish the model. Once the decals were dry and all adhesive stains cleaned up, I tried spraying a coat of Gunze Mr. Color Clear Gloss in an attempt to cure the silvering, but no dice. The final coat was a semi-gloss coat (Gunze Clear Gloss and Flat base mix) to give the model an appropriate matt finish.

| Click on images below to see larger images |  |
| --- | --- |
| Steve Bamford will insert a click-able smaller version of photo 07.jpg here. Clicking on that photo will bring up the full sized version of photo 07.jpg | Steve Bamford will insert a click-able smaller version of photo 08.jpg here. Clicking on that photo will bring up the full sized version of photo 08.jpg |

After that, it was a matter of just putting together the various sub-assemblies and the model was complete. Paper glue was used to attach all clear plastic parts such as the HUD, windscreen and tail lights.

Weathering was limited to just a wash with dilute India ink in certain areas, and patches of FS36118 sprayed on the matt clear coat to highlight a few worn access panels.

All-in-all the finished TUDM Hornet was a reasonable looking model, but never again will I try this. A lot of time could have been saved by starting with the Hasegawa kit, especially as far as where the dropped flaps were concerned. But I wanted to keep the cost to a minimum and at the same time develop modelling skills without risking an expensive kit.

Jason Cheah

www.F-4DableModels.com

| Click on images below to see larger images |  |
| --- | --- |
| Steve Bamford will insert a click-able smaller version of photo 09.jpg here. Clicking on that photo will bring up the full sized version of photo 09.jpg | Steve Bamford will insert a click-able smaller version of photo 10.jpg here. Clicking on that photo will bring up the full sized version of photo 10.jpg |

"Photographs by KT Goh and Andrew Siew."

Photos and text © 2003 by " Jason Cheah"